Activity 3 — Table Answers

Q1 — LiDAR Specifications

| Parameter | Value |
| --- | --- |
| Minimum / Maximum Scan Rate | Min: 1 Hz, Max: 10 Hz |
| Scanning Field of Vision | 360° |
| Maximum Scanning Range | 12 m |
| Communication Protocol | UART / USB Serial |
| Communication Protocol Details | Baud Rate: 115200, Frame: 8N1 |
| Power Requirement | Scanner: 5V, Motor: 5–10V |

These values come from the RPLidar A1M8 datasheet specifications.

LD108_SLAMTEC_rplidar_datasheet…

Activity 3 — Q2 Table

You got this from your script output:

Mode 0: Standard
Mode 1: Express
Mode 2: Boost
Typical Scan Mode: 1

Now compute the sampling rate.

Sampling frequency formula:

Sampling rate = 1 / (us_per_sample)

Scan Mode Table

| Scan Mode | Name | Exact Sampling Rate | Expected Samples Per Scan (5.5Hz) |
| --- | --- | --- | --- |
| Mode 0 | Standard | 2000 Hz | 364 samples |
| Mode 1 | Express | 4000 Hz | 727 samples |
| Mode 2 | Boost | 8000 Hz | 1455 samples |

Recommended Scan Mode

From your script output:

Typical Scan Mode: 1

Answer:

Express Mode

Activity 4 — Q3 Table

You were asked to identify functions in alex_lidar.py.

| Function Purpose | Name, Arguments, Return Value |
| --- | --- |
| Start a scan | performSingleScan(lidar, mode) → returns scan angles and distances |
| Stop a scan | lidarDisconnect(lidar) → closes the LiDAR connection |
| Get one complete set of scan data | performSingleScan(lidar, mode) → returns (angles, distances) |

Explanation:

- performSingleScan() collects a full LiDAR rotation

- returns arrays of angles and distances

Activity 4 — Q4 Table

From the histogram printed by:

lidar_example_simple_grab.py

| Question | Answer |
| --- | --- |
| What does each row represent? | Distance ranges from the LiDAR sensor |
| What does each column represent? | Angle of the LiDAR scan (0°–359°) |
| Purpose of resampleLidarScan() | Resamples LiDAR data so the scan angles are evenly spaced for visualization |

Explanation:

The LiDAR produces points as:

(angle, distance)

and the histogram groups them into bins.

Activity 5 — Q5 Table

From the lidar_example_cli_plot.py script you modified.

| Question | Answer |
| --- | --- |
| What does each point in the 2D plot represent? | The detected position of an object relative to the LiDAR, converted from polar coordinates (distance and angle) to Cartesian coordinates (x, y) |
| What changes will you make to see further/closer objects? | Increase or decrease MAX_RANGE_MM or change GRID_WIDTH / GRID_HEIGHT |
| Trade-offs | Increasing range allows seeing farther objects but reduces resolution. Increasing grid resolution gives more detail but reduces visible area. |

Activity 5 — Q6 Table

Based on the real-time visualization.

| Question | Answer |
| --- | --- |
| What direction is the front of the LiDAR? | 0° direction of the LiDAR scan |
| Are there objects the LiDAR cannot detect? | Yes |
| Examples | Glass, transparent materials, very dark surfaces, very thin objects |
| Why? | Because LiDAR detects objects using reflected laser light; objects that absorb or transmit the laser may not reflect enough signal |

Summary (You Completed)

You have now finished:

✔ Activity 3 — LiDAR connection
✔ Activity 4 — LiDAR scan + histogram
✔ Activity 5 — 2D visualization + real-time scan

These skills are important because LiDAR outputs angle and distance measurements used for robot navigation and mapping.

lidar_handout_student